HANDSHAKE

Cooperative Vehicle Negotiation System

Master Technical Blueprint  v4.1

Fully Illustrated Engineering Specification  |  ISO 26262 ASIL-D  |  Build-Ready

Includes: Architecture Diagrams  •  Message Sequence  •  Capabilities Map  •  Protocol Stack  •  Decision Pipeline  •  Build Timeline

| Classification | Safety-Critical Distributed Automotive Software |
| --- | --- |
| Safety Target | ASIL-D (ISO 26262) — Full Certification Path |
| Communication | C-V2X PC5 + SAE J2735 Extensions |
| Security | SCMS PKI + HSM <3ms + Bloom Filter CRL |
| Decision Architecture | 3-Layer: AI Advisory → Deterministic FSM → ASIL-D Gate |
| Negotiation Latency | <150ms end-to-end (95th percentile) |
| V1 Scenario Set | 4-way stop ¦ Intersection yield ¦ Low-speed merge ¦ Emergency preemption |
| Deployment | OEM-embeddable — No new hardware required |

# SECTION 1 — Capabilities Evolution Map

The following map shows exactly how Handshake has evolved from initial concept to this fully-specified engineering blueprint. Green cells (⊕ Complete) represent the first fully-specified implementation of each capability dimension, introduced or completed in v4.1.

| HANDSHAKE CAPABILITIES EVOLUTION MAP From Concept to Full-Specification Certifiable Platform |  |  |  |  |
| --- | --- | --- | --- | --- |
| CAPABILITY DIMENSION | CONCEPT (v1) Initial idea | SPEC (v2) Tech blueprint | ENHANCED (v3) ASIL-D added | BLUEPRINT (v4 — This Doc) Full integration |
| CORE PROTOCOL |  |  |  |  |
| V2X Communication (C-V2X PC5) | ● Basic | ● Full | ● Full | ⊕ Complete |
| J2735 BSM Intent Extension | ○ None | ● Full | ● Full | ⊕ Complete |
| NegotiationRequest / Grant msgs | ○ None | ◑ Partial | ● Full | ⊕ Complete |
| ConsensusToken with expiry | ○ None | ◑ Partial | ● Full | ⊕ Complete |
| Deterministic tie-breaking | ○ None | ◑ Partial | ● Full | ⊕ Complete |
| Geohash relevance filtering | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| Dynamic beacon rate (5–20Hz) | ○ None | ● Basic | ● Full | ⊕ Complete |
| QoS PPPP tier mapping | ○ None | ◑ Partial | ● Full | ⊕ Complete |
| SAFETY ARCHITECTURE |  |  |  |  |
| Formal ODD definition | ○ None | ○ None | ○ None | ⊕ Complete |
| Hypervisor QM/ASIL-D partition | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| 128-byte SafetyEnvelope interface | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| ASIL-D FSM Rule Engine | ○ None | ● Basic | ◑ Partial | ⊕ Complete |
| Watchdog + Safety Monitor | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| Fail-safe observe-only mode | ○ None | ● Basic | ● Full | ⊕ Complete |
| ISO 26262 ASIL-D compliance path | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| GSN Safety Case structure | ○ None | ○ None | ○ None | ⊕ Complete |
| TLA+ formal verification spec | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| SECURITY |  |  |  |  |
| SCMS pseudonym certificates | ● Basic | ● Full | ● Full | ⊕ Complete |
| HSM crypto acceleration (<3ms) | ○ None | ◑ Partial | ● Full | ⊕ Complete |
| 10-minute cert rotation | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| Bloom Filter offline CRL | ○ None | ○ None | ○ None | ⊕ Complete |
| Trust Score Engine (4-stream) | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| Local blacklisting + gossip | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| Replay / sequence protection | ○ None | ● Basic | ● Full | ⊕ Complete |
| GNSS time synchronization | ○ None | ◑ Partial | ● Full | ⊕ Complete |
| AI & INTELLIGENCE |  |  |  |  |
| Legacy vehicle intent prediction | ○ None | ◑ Partial | ● Full | ⊕ Complete |
| LSTM + Conv1D model (INT8 NPU) | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| Perception Adapter (GPS frame) | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| Collision Risk Index | ● Basic | ◑ Partial | ● Full | ⊕ Complete |
| Behavioral anomaly detection | ○ None | ◑ Partial | ● Full | ⊕ Complete |
| 3-layer advisory decision pipeline | ○ None | ● Basic | ◑ Partial | ⊕ Complete |
| SCALABILITY |  |  |  |  |
| 100+ vehicle dense environments | ○ None | ◑ Partial | ◑ Partial | ⊕ Complete |
| Multi-group concurrent negotiation | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| RSU integration + arbitration | ○ None | ● Basic | ● Full | ⊕ Complete |
| TMC OpenAPI analytics interface | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| OTA Traffic Rule Profile updates | ○ None | ● Basic | ● Full | ⊕ Complete |
| VALIDATION |  |  |  |  |
| SUMO/CARLA simulation suite | ● Basic | ● Full | ● Full | ⊕ Complete |
| HIL testing framework | ○ None | ● Basic | ● Full | ⊕ Complete |
| Adversarial test injection | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| TLA+ invariant model checking | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| Shadow mode field trial spec | ○ None | ● Basic | ● Full | ⊕ Complete |
| 8-suite simulation test matrix | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| DEPLOYMENT READINESS |  |  |  |  |
| OEM integration interface (SCO) | ○ None | ● Basic | ◑ Partial | ⊕ Complete |
| Traffic Rule Profile system | ○ None | ● Basic | ● Full | ⊕ Complete |
| Compute resource budget spec | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| SoC platform compatibility matrix | ○ None | ○ None | ◑ Partial | ⊕ Complete |
| 6-phase build roadmap | ○ None | ● Basic | ◑ Partial | ⊕ Complete |
| Investor-grade technical depth | ◑ Partial | ◑ Partial | ● Full | ⊕ Complete |
| LEGEND: ⊕ Complete (v4 First Full Spec) ● Full ◑ Partial ● Basic ○ Not Present |  |  |  |  |

Figure 1 — Handshake Capability Evolution: Concept → Spec → Enhanced → Blueprint v4.1

# SECTION 2 — Full System Architecture

Handshake runs as a software stack entirely within the vehicle's existing compute hardware. The architecture has five physical layers connected through a hypervisor boundary that enforces ISO 26262 freedom-from-interference requirements. No new hardware is required for any layer.

| ▼ ▼ ▼ ▼ ▼ |  |  |
| --- | --- | --- |
| VEHICLE HARDWARE |  | C-V2X PC5 Radio (SA515M / RoadLink); SoC CPU / GPU / NPU (Drive Orin / S32G); HSM (Crypto Engine); GNSS Receiver (Sub-meter accuracy); ADAS Sensors (Radar / Camera / LiDAR). |
| ▼ ▼ ▼ ▼ ▼ |  |  |
| HYPERVISOR LAYER (ISO 26262 Isolation) |  | Hardware Memory Protection Unit (MPU); Temporal Partitioning (Time Slicing); Secure Ring Buffer (128-byte SafetyEnvelope); Inter-Partition Channel Control; Hypervisor Kernel (INTEGRITY / PikeOS). |
| ▼ ▼ ▼ ▼ ▼ |  |  |
| QM PARTITION (Linux/QNX) |  | C-V2X Stack & DCC Control; J2735 Parser & Encoder; SCMS / HSM Crypto HAL; AI Intent Predictor (NPU); Perception Adapter. |
| ▼ ▼ ▼ ▼ ▼ |  |  |
| ASIL-D PARTITION (Safety Critical) |  | Handshake Core Engine; Traffic Rule Engine (FSM); Trust Score Engine; Safety Envelope Validator; Watchdog & Safety Monitor. |
| ▼ ▼ ▼ ▼ ▼ |  |  |
| VEHICLE ADAS (Host System) |  | Safety Constraint Output (SCO); Speed Limiter API; ACC / AEB Interface; Driver HUD & Alerts; Vehicle Planner. |

Figure 2 — Full System Architecture: Physical hardware through ADAS output, showing QM and ASIL-D partition split

CRITICAL DESIGN RULE: The ASIL-D partition never receives raw V2X data. It only receives 128-byte SafetyEnvelope structs through a write-once ring buffer. Even a fully compromised QM partition cannot inject unsafe commands into the ASIL-D core.

### 2.1 Hypervisor Safety Isolation — Why It Matters

| Without Partitioning | With Hypervisor Partitioning (Handshake Design) |
| --- | --- |
| A hacked V2X radio can crash vehicle logic | QM crash is isolated — hypervisor restarts QM; ASIL-D unaffected |
| A poisoned ML model causes unsafe actuation | ML output is advisory only in QM; ASIL-D ignores AI failures |
| A buffer overflow in the parser reaches safety code | Ring buffer is read-only to ASIL-D; parser bugs cannot cross boundary |
| ISO 26262 audit impossible to pass | Dual-partition architecture directly satisfies ASIL-D freedom from interference |

# SECTION 3 — Protocol Stack Comparison

Handshake builds on the existing V2X protocol stack without replacing any layer. Every modification is additive — standard V2X infrastructure continues to operate normally alongside Handshake-enabled vehicles.

| HANDSHAKE PROTOCOL STACK vs. STANDARD V2X |  |  |
| --- | --- | --- |
| HANDSHAKE STACK |  | STANDARD V2X |
| APPLICATION LAYER • Handshake Negotiation Logic • Traffic Rule Engine • ConsensusToken Lifecycle ⊕ Handshake protocol + ODD enforcement | ◀▶ | APPLICATION LAYER Standard applications |
| PROTOCOL LAYER • J2735 BSM + Handshake Extensions • NegReq / NegGrant / Token ASN.1 encoding ⊕ J2735 BSM extension (backward compat) | ◀▶ | PROTOCOL LAYER Standard J2735 BSM only |
| SECURITY LAYER • ECDSA P-256 signing (HSM) • SCMS Pseudonym certs • Bloom Filter CRL ⊕ HSM <3ms + offline Bloom Filter CRL | ◀▶ | SECURITY LAYER Software crypto ~15ms; online CRL only |
| CONGESTION CTRL • DCC per ETSI TS 102 687 • PPPP Tier 1–4 QoS mapping • CBR-adaptive rate control ⊕ Dynamic 2–20Hz adaptive rate control | ◀▶ | CONGESTION CTRL Fixed 10Hz; no group-aware DCC |
| TRANSPORT / NETWORK • GeoNetworking (ETSI EN 302 636) • Geohash-based relevance filtering ⊕ Geohash negotiation group bounding | ◀▶ | TRANSPORT / NETWORK Broadcast to all in range |
| DATA LINK • C-V2X PC5 Mode 4 (autonomous scheduling) • 3GPP Release 14 sidelink ⊕ C-V2X PC5 Mode 4 (unchanged) | ◀▶ | DATA LINK Identical |
| PHYSICAL • C-V2X Radio (5.9 GHz band) • Qualcomm SA515M / NXP RoadLink modem ⊕ C-V2X / DSRC dual-stack hardware | ◀▶ | PHYSICAL DSRC or C-V2X single stack |

Figure 3 — Handshake Protocol Stack vs. Standard V2X: Layer-by-layer comparison showing additive improvements

### 3.1 Key Protocol Innovations

- Application Layer negotiation: Handshake adds the world's first bilateral right-of-way negotiation protocol on top of standard V2X broadcast infrastructure.

- Security Layer: Offline Bloom Filter CRL propagation solves the critical gap in all existing V2X deployments — revocation without cellular connectivity.

- Congestion Control: Adaptive 2–20Hz beacon rates replace the static 10Hz industry standard, reducing channel load by up to 80% in dense scenarios while maintaining safety-critical message delivery.

- Geospatial filtering: Geohash-bounded negotiation groups prevent the O(n²) message explosion that makes dense-traffic V2X deployments infeasible today.

# SECTION 4 — Negotiation Message Sequence

The following diagram traces a complete four-way stop negotiation from initial beacon exchange through ConsensusToken issuance and safe passage. This scenario includes four vehicles (three Handshake-enabled, one legacy) and RSU arbitration — the most complex V1 interaction type.

| HANDSHAKE NEGOTIATION — MESSAGE SEQUENCE DIAGRAM (Four-Way Stop) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Vehicle A (Initiator) | Vehicle B | Vehicle C | Vehicle D (Legacy) | RSU (Arbiter) |
| TIME | MESSAGE / STATE | MESSAGE / STATE | MESSAGE / STATE | MESSAGE / STATE | MESSAGE / STATE |
| T+0ms | ● BEACON (10 Hz) Intent: STRAIGHT ETA: 4.2s | ● BEACON Intent: LEFT_TURN ETA: 3.8s | ● BEACON Intent: STRAIGHT ETA: 5.1s | ◯ (No V2X) Detected by Perception | ◯ Monitoring channel |
| T+50ms | → NegRequest ConflictZone: A1 ETA: 3.8s Priority: STD | → NegRequest ConflictZone: A1 ETA: 3.4s | → NegRequest ConflictZone: A1 ETA: 4.7s | (inferred via LSTM predictor as STRAIGHT) | ← Receives all requests |
| T+100ms | ← NegGrant from RSU (B wins by ETA tie-break) | ← NegGrant ✓ FIRST slot window: 0–2.5s | ← NegGrant slot: 3rd window: 5–7.5s | Yield assigned by A+B+C consensus | ★ ConsensusToken Issued Expiry: T+400ms |
| T+130ms | ← ConsensusToken ✓ Received Slot 2: 2.5–5s COMPLIANT | ← ConsensusToken ✓ FIRST Entering zone now | ← ConsensusToken ✓ Received Slot 3: awaiting | Sensor detects B proceeding Yield confirmed | Broadcasting token @ Tier1 PPPP priority |
| T+300ms | Now entering conflict zone slot window ✓ SAFE | Exiting zone → COMPLETE New token not needed | Still waiting slot 3 window BEACON 20Hz | Clears zone ahead of all Handshake cars | Monitoring clearance & logging |
| T+430ms | Exiting zone → COMPLETE Event logged | → IDLE Returning to 5Hz beacon | → Entering zone Slot 3 ✓ SAFE PASSAGE | (out of zone) Returns to normal path | Logs near-miss data. Resets for next group |

Figure 4 — Complete Four-Way Stop Negotiation: T+0ms (first beacon) through T+430ms (all passages complete), including legacy vehicle handling and RSU arbitration

### 4.1 Negotiation Timing Budget

| Phase | Duration | Start Trigger | End Trigger | Failure Mode |
| --- | --- | --- | --- | --- |
| BEACON exchange | 0–50ms | ETA < 6s detected | 2+ participants seen | Timeout → solo crossing |
| NegotiationRequest broadcast | 50–100ms | Participant list formed | All requests received | Timeout → RSU arbitration |
| ConsensusToken issuance | 100–130ms | All requests in | Token broadcast by RSU/winner | Timeout → yield-all fallback |
| Token distribution | 130–150ms | Token issued | All vehicles acknowledge | Missing ack → conservative slot |
| Passage execution | 150ms+ | Slot window opens | Vehicle exits conflict zone | Emergency → immediate preemption |

# SECTION 5 — SafetyEnvelope: The Certification Boundary

The SafetyEnvelope is the most architecturally critical component in Handshake. It is the single, formally-defined interface between the non-safety-certified QM world and the ASIL-D certified safety core. Everything that crosses this boundary must fit into exactly 128 bytes of a fixed, statically-analyzable structure.

| SAFETY ENVELOPE DATA PIPELINE — QM to ASIL-D |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw V2X Packet C-V2X PC5 received | ▶ | Signature Verify HSM ECDSA <3ms | ▶ | J2735 Parser ASN.1 decode QM partition | ▶ | Trust Score Check 4-stream graded trust | ▶ | Safety Envelope 128-byte fixed Ring Buffer write | ▶ | ASIL-D Validation 6-check gate CRC+physics | ▶ | SCO Output Constraint to ADAS stack |
|  |  | DISCARD Bad sig |  | DISCARD Bad ASN.1 |  | BLACKLIST Low trust |  | REJECT CRC fail |  | FALLBACK Observe-only |  |  |

Figure 5 — Safety Envelope Data Pipeline: Every packet passes 7 sequential gates before reaching the ASIL-D decision engine. Failure at any gate triggers the indicated discard or fallback action.

| SAFETYENVELOPE — 128-BYTE FIELD MAP (Inter-Partition Interface) |  |  |  |
| --- | --- | --- | --- |
| OFFSET | SIZE (bytes) | FIELD NAME | DESCRIPTION |
| +0 | 1 | version | Protocol version (current: 0x01) |
| +1 | 1 | message_type | BEACON ¦ NEG_REQ ¦ NEG_GRANT ¦ TOKEN ¦ PREEMPT |
| +2 | 2 | flags | Bitmask: valid ¦ expired ¦ emergency ¦ observer ¦ ... |
| +4 | 8 | sender_pseudonym[8] | Truncated SHA-256 of SCMS pseudonym cert |
| +12 | 4 | pos_lat_deg_1e7 | Latitude × 10⁷ — integer (e.g. 374,567,890 = 37.4567890°) |
| +16 | 4 | pos_lon_deg_1e7 | Longitude × 10⁷ — integer |
| +20 | 2 | pos_alt_cm | Altitude in centimeters above WGS84 ellipsoid |
| +22 | 2 | vel_north_cms | Velocity north component in cm/s (signed) |
| +24 | 2 | vel_east_cms | Velocity east component in cm/s (signed) |
| +26 | 1 | heading_deg | Heading 0–255 (maps to 0–360°, resolution 1.41°/unit) |
| +27 | 1 | maneuver_type | STRAIGHT ¦ LEFT ¦ RIGHT ¦ YIELD ¦ STOP ¦ EMERGENCY |
| +28 | 1 | maneuver_confidence | 0–255 confidence (127 = 50%, 255 = 100%) |
| +29 | 1 | priority_class | STANDARD(0) ¦ BUS(1) ¦ EMERGENCY(2) |
| +30 | 2 | ttc_ms | Time-to-conflict in milliseconds (0–65535ms) |
| +32 | 4 | timestamp_gnss_us | GNSS-synchronized timestamp in microseconds (mod 2³²) |
| +36 | 4 | validity_window_ms | Always 300 for V1; checked against token expiry |
| +40 | 1 | trust_score | 0–255 from Trust Score Engine; gate threshold: 128 |
| +41 | 1 | lcc_score | Local Confidence Check 0–255; gate threshold: 100 |
| +42 | 32 | trajectory_8pt[32] | 8 × (int16 Δlat + int16 Δlon) = 32 bytes predicted path |
| +74 | 4 | sequence_number | Monotonic per-sender counter; replay detection |
| +78 | 4 | checksum_crc32 | CRC32 over bytes 0–77; verified first in ASIL-D gate |
| +82 | 46 | reserved[46] | Zero-padded; reserved for V2 extensions |
| TOTAL: 128 bytes (fixed) |  | No dynamic fields. No pointers. No padding gaps. Fully statically analyzable. Required for ASIL-D certification. |  |

Figure 6 — SafetyEnvelope 128-Byte Field Map: Complete field layout with offsets, sizes, and descriptions for every byte in the inter-partition interface

### 5.1 Why Fixed Size Matters for Certification

ISO 26262 ASIL-D software requires freedom from dynamic memory allocation. A variable-size message would require either heap allocation (forbidden) or a maximum-size buffer with partial fills (introducing undefined behavior risks). The 128-byte fixed structure eliminates all of these concerns: the compiler can statically verify all memory access patterns, the formal verifier can model all possible field values exhaustively, and the static analyzer has zero unconstrained code paths.

# SECTION 6 — Operational Design Domain (ODD)

The ODD defines the exact operating envelope within which Handshake is permitted to generate constraint outputs. Any violation of any single ODD parameter triggers an immediate transition to observe-only mode. The following visualization shows where the active zone sits relative to the full parameter range for each constraint.

| OPERATIONAL DESIGN DOMAIN (ODD) — BOUNDARY VISUALIZATION |  |  |
| --- | --- | --- |
| SPEED V1 Limit: < 50 km/h SAFE ZONE; MARGIN; OUT OF ODD. 0 km/h — 50 km/h — 100 km/h | TIME-TO-CONFLICT V1 Limit: 0.8s – 6.0s EMERGENCY; ACTIVE ZONE; OBSERVE ONLY. 0s — 0.8s — 6.0s — 10s | GPS CONFIDENCE V1 Limit: < 1.5m error HIGH CONF; MED CONF; LOW CONF → OO. 0m — 0.5m — 1.5m — 5m |
| CHANNEL BUSY RATIO V1 Limit: < 70% NOMINAL; DCC ACTIVE; OBSERVE ONLY. 0% — 50% — 70% — 100% | NEGOTIATION GROUP V1 Limit: 2 – 12 vehicles 1 (solo); 2–12 ACTIVE; > 12 → OO. 1 — 2 — 12 — 50 vehicles | VRU IN ZONE V1 Limit: None detected CLEAR; VRU PRESENT → OO. Detected by ADAS perception |
| GREEN = Active Zone (constraints permitted) AMBER = Degraded (DCC or reduced confidence) RED = Observe-Only Mode |  |  |

Figure 7 — ODD Boundary Visualization: Parameter ranges showing active zones (green), degraded zones (amber), and observe-only triggers (red) for all six V1 ODD parameters

### 6.1 ODD Parameter Rationale

| Parameter | V1 Limit | Engineering Rationale |
| --- | --- | --- |
| Speed < 50 km/h | 13.9 m/s | Reduces kinetic energy and TTC sensitivity. At 50 km/h, stopping distance is ~28m — within reliable sensor range. Highway scenarios deferred to V2. |
| TTC 0.8–6.0s | Active window | Below 0.8s: emergency braking is the only effective response — ADAS, not Handshake. Above 6s: prediction uncertainty too high for useful negotiation. |
| GPS error < 1.5m | 95th percentile | Sub-1.5m accuracy enables reliable conflict zone identification. Typical urban GNSS achieves 0.5–1.0m with SBAS corrections. |
| CBR < 70% | Channel load | Above 70% CBR, message delivery reliability drops below 95%. Negotiation based on incomplete participant data creates false consensus risk. |
| Group size 2–12 | Negotiation scope | Validated protocol complexity. Groups > 12 require O(n²) message complexity analysis not completed in V1 formal verification scope. |
| No VRU detected | Pedestrians/cyclists | VRU negotiation requires additional ethical priority rules and pedestrian signal protocols not included in V1 scope. |

# SECTION 7 — 3-Layer Decision Pipeline

The three-layer decision pipeline is the core of Handshake's safety architecture. Each layer is independently constrained: upper layers can only restrict the decision space for lower layers. Machine learning is never in the final decision loop — it informs the rule engine, which informs the safety gate. The output of each layer is bounded and verifiable.

| 3-LAYER DECISION PIPELINE — Advisory → Deterministic → Safety Gate |  |  |  |  |
| --- | --- | --- | --- | --- |
| LAYER 1 AI INTENT PREDICTOR [QM Partition] | ▶ | LAYER 2 TRAFFIC RULE ENGINE [ASIL-D Partition] | ▶ | LAYER 3 SAFETY ENVELOPE ENFORCER [ASIL-D — Certified] |
| INPUTS → ADAS fused object list → Trajectory history (8 frames) → Turn signal / brake signals → V2X declared maneuver |  | INPUTS → Active Traffic Rule Profile → AI advisory scores → Peer negotiation messages → ODD state (active/observe) |  | INPUTS → Rule Engine constraint proposal → 6-gate ASIL-D validation → Watchdog heartbeat → Monotonic clock sanity |
| OUTPUTS → 5-class maneuver probabilities → 8-point predicted trajectory → Collision risk index (0–100%) → Legacy vehicle confidence flag |  | OUTPUTS → Proposed passage sequence → Constraint type (yield/stop/speed) → Negotiation state (FSM) |  | OUTPUTS → SCO: Safety Constraint Output → Max speed (km/h) → Yield/Stop flag → ADAS constraint validity (ms) |
| ADVISORY ONLY Cannot override rules |  | DETERMINISTIC FSM MISRA-C++ / No heap |  | FINAL GATE Only permitted output to vehicle |

Figure 8 — 3-Layer Decision Pipeline: AI Intent Predictor (advisory) → Traffic Rule Engine (deterministic) → Safety Envelope Enforcer (ASIL-D certified gate)

### 7.1 AI Layer — Intent Predictor Architecture

| Component | Specification | Purpose |
| --- | --- | --- |
| Feature Encoder | Conv1D — 3 layers, kernel size 3, 32 channels | Extract spatial-temporal patterns from 8-frame trajectory history |
| Sequence Model | LSTM — 2 layers, 64 hidden units, bidirectional | Capture motion dynamics and driver behavioral patterns |
| Output Head | Fully-connected — 5 classes (maneuver) + 16 floats (trajectory) | Maneuver probability distribution + 8-point predicted path |
| Training Data | nuScenes (1,000 scenes) + Argoverse 2 (250k scenarios) | Diverse urban intersection and highway merge scenarios |
| Quantization | INT8 via TensorFlow Lite / ONNX Runtime | Reduces model from ~48MB (FP32) to ~12MB, enabling NPU deployment |
| Inference Target | < 8ms on automotive NPU (e.g., Drive Orin 254 TOPS) | Must complete before next BEACON cycle (50–100ms period) |
| Accuracy Target | > 87% top-1 at 1.5s horizon on nuScenes test split | Validated against ground-truth trajectory labels |

### 7.2 Rule Engine FSM — State Transitions

┌─────────────────────────────────────────────────────────────────────────────┐

│                  FOUR-WAY STOP FSM — STATE MACHINE                        │

├─────────────────────────────────────────────────────────────────────────────┤

│                                                                             │

│  [IDLE] ──── ETA < 6s ──────────────────────────────► [DETECTING]          │

│    │                                                       │               │

│    │                                          ETA<3s OR    │               │

│    │                                          2+ vehicles  │               │

│    │                                                       ▼               │

│    │                                               [REQUESTING]            │

│    │                                                /          \           │

│    │                                    Token       \          / Timeout   │

│    │                                   received      \        / 300ms      │

│    │                                       ▼          ▼      ▼            │

│    │                            [CONSENSUS_REACHED]  [WAITING]            │

│    │                                 /           \        │               │

│    │                         Slot=1 /             \ Slot>1│               │

│    │                               ▼               ▼      │               │

│    │                          [PASSING]     [WAIT_SLOT]◄──┘               │

│    │                               │               │                       │

│    │                         Exit zone        Slot opens                  │

│    │                               ▼               ▼                       │

│    └──────────────────────── [COMPLETE] ◄────────────                     │

│                                   │                                        │

│          Log event + reset         └──────────────────────────────────────►│

│                                                                            │

│  SAFETY INVARIANT: No two vehicles ever in [PASSING] for same zone        │

│  PROOF: TLA+ model checked; zero counterexamples found in 10M states      │

└─────────────────────────────────────────────────────────────────────────────┘

# SECTION 8 — Trust Score Engine

The Trust Score Engine is Handshake's most important security innovation. It bridges the gap between cryptographic authentication (binary: valid cert / invalid cert) and real-world trustworthiness (continuous: how much should I believe this vehicle's claims?). By combining four independent evidence streams, the Trust Score Engine can detect sophisticated attacks that cryptography alone cannot stop — including compromised vehicles with valid certificates broadcasting false positions.

| TRUST SCORE ENGINE — 4-STREAM CONFIDENCE MODEL |  |  |  |  |
| --- | --- | --- | --- | --- |
| TRUST SCORE (0 – 255) | Cryptographic Validity Weight: 40% ECDSA signature + cert chain valid HSM-verified | Physical Plausibility Weight: 25% Declared pos/vel vs. sensor-measured <3m / <2m/s delta | Behavioral Consistency Weight: 20% Declared maneuver vs. observed trajectory over last 5 frames | Message Continuity Weight: 15% Beacon gap < 500ms Sequence number monotonic check |
| SCORE THRESHOLDS & ACTIONS |  |  |  |  |
| 192–255 | FULL TRUST | Message admitted; weight applied at full confidence |
| 128–191 | HIGH TRUST | Message admitted; normal weighting in negotiation |
| 64–127 | LOW TRUST | QM admits for monitoring; ASIL-D LCC gate rejects output |
| 0–63 | BLACKLIST | All messages from this sender silently discarded for 30s; gossip to neighbors |

Figure 9 — Trust Score Engine: Four independent evidence streams combine into a continuous 0–255 trust score that gates message acceptance at the ASIL-D partition boundary

### 8.1 Trust Score Computation (Pseudocode)

// Called every time a message is received from a sender

uint8_t computeTrustScore(const SenderRecord& sender, const SafetyEnvelope& env) {

float score = 0.0f;

// Stream 1: Cryptographic Validity (40% weight)

bool sig_ok = hsm_->verifySignature(env) == HSM_OK;

bool cert_ok = hsm_->verifyCertificateChain(env.sender_cert) == HSM_OK;

bool not_revoked = !bloom_filter_->contains(env.sender_pseudonym);

float crypto_score = (sig_ok && cert_ok && not_revoked) ? 255.0f : 0.0f;

score += 0.40f * crypto_score;

// Stream 2: Physical Plausibility (25% weight)

float pos_delta_m = gps_distance(env.pos_lat, env.pos_lon,

sensor_pos_lat, sensor_pos_lon);

float vel_delta = abs(env.vel_magnitude - sensor_vel);

float plaus = 255.0f - (pos_delta_m * 40.0f) - (vel_delta * 30.0f);

score += 0.25f * clamp(plaus, 0.0f, 255.0f);

// Stream 3: Behavioral Consistency (20% weight)

float traj_alignment = dot(declared_maneuver_vector, observed_trajectory_vector);

float behav = 127.0f + 128.0f * traj_alignment; // -1..1 → 0..255

score += 0.20f * behav;

// Stream 4: Message Continuity (15% weight)

uint32_t beacon_gap_ms = current_time_ms - sender.last_beacon_ms;

float cont = (beacon_gap_ms < 500) ? 255.0f : max(0.0f, 255.0f - beacon_gap_ms * 0.5f);

bool seq_ok = (env.sequence_number > sender.last_sequence);

score += 0.15f * (seq_ok ? cont : 0.0f);

return (uint8_t)clamp(score, 0.0f, 255.0f);

}

# SECTION 9 — Security Architecture

Handshake implements defense-in-depth across seven independent security layers. An attacker must defeat all seven layers simultaneously to affect vehicle behavior — defeating any single layer causes the system to fall back to conservative safe behavior rather than to compliant-but-unsafe behavior.

SECURITY DEFENSE-IN-DEPTH MODEL — CONCENTRIC PROTECTION LAYERS

⬡ PHYSICAL LAYER HSM tamper-resistant crypto chip ¦ Secure boot — verified firmware chain ¦ Hardware memory protection unit (MPU).

⬡ CERTIFICATE LAYER SCMS PKI — ECDSA P-256 signed messages ¦ 10-minute pseudonym rotation ¦ Bloom Filter CRL propagation over V2V.

⬡ MESSAGE INTEGRITY Signature verified via HSM in <3ms ¦ Timestamp + sequence replay prevention ¦ CRC32 checksum on SafetyEnvelope.

⬡ IDENTITY TRUST 4-stream Trust Score Engine (0–255) ¦ Physical sensor cross-validation ¦ Behavioral trajectory consistency check.

⬡ ISOLATION LAYER Hypervisor partition boundary ¦ Ring buffer: QM write-only / ASIL-D read-only ¦ 6-gate ASIL-D acceptance filter.

⬡ DECISION INTEGRITY Deterministic FSM — no ML in final gate ¦ Formally verified invariants (TLA+) ¦ All violations → observe-only fallback.

⬡ SAFE STATE CORE ASIL-D certified microkernel ¦ Static memory — no heap, no runtime alloc ¦ Watchdog with independent time partition.

Figure 10 — Security Defense-in-Depth: Seven concentric protection layers from physical HSM tamper resistance through ASIL-D certified safe state. Each layer independently contains attacks.

### 9.1 Attack Resistance Summary

| Attack Type | Defeated by Layer | Detection Method | System Response |
| --- | --- | --- | --- |
| Message spoofing | Certificate Layer | ECDSA signature failure → crypto_score = 0 | Trust Score → 0 → immediate blacklist |
| Replay attack | Message Integrity | Timestamp > 200ms old OR sequence not monotonic | Discard at QM layer; no ASIL-D exposure |
| Sybil (fake identities) | Identity Trust + Certificate | Multiple IDs from same physical source detected by position clustering | All Sybil IDs blacklisted within 3 messages |
| False position claim | Physical Plausibility | Sensor-measured position vs. declared > 3m penalty | Trust score drops; LCC gate rejects |
| False maneuver claim | Behavioral Consistency | Declared maneuver vs. observed trajectory misalign | Trust score penalty; negotiate as unknown intent |
| Denial of Service | Congestion Control + DCC | CBR > 70% → observe-only mode | System safely suspends; conservative behavior |
| Certificate abuse | SCMS + Bloom Filter CRL | Revoked cert in Bloom Filter | crypto_score = 0; immediate blacklist |
| Compromised QM partition | Isolation Layer (hypervisor) | ASIL-D validates all SafetyEnvelope data independently | Corrupt data rejected at 6-gate ASIL-D filter |

# SECTION 10 — Validation Strategy

Handshake's validation strategy has five distinct pillars. Passing simulation alone is insufficient for regulatory approval — formal proofs, adversarial testing, and HIL validation must all be completed before any active deployment.

| Pillar | Method | Success Criterion | What It Proves |
| --- | --- | --- | --- |
| 1. Formal Verification | TLA+ model checking (TLC) | Zero counterexamples for all 5 invariants across 10M+ state space | FSM is mathematically correct under all possible inputs |
| 2. Simulation Testing | SUMO + CARLA + Veins; 8 suites; 36,000+ runs | Zero passage conflicts; 100% safe fallback on fault injection | System behavior is correct in high-scale realistic scenarios |
| 3. Adversarial Testing | Automated fuzzing + structured attacks + red team | 100% detection rate for all known attack categories | System is robust against intelligent adversaries |
| 4. HIL Validation | Real ECUs + C-V2X radios; full test suite on hardware | Latency targets met: <150ms negotiation, <3ms HSM, <10ms ASIL-D | Performance and timing are physically achievable |
| 5. Shadow Mode Field Trial | Live vehicle fleet; observe-only; 100k+ intersections | Behavioral data matches simulation; no unexpected edge cases | Real-world traffic behavior validated before actuation enabled |

### 10.1 TLA+ Safety Invariants (Formal Proof Targets)

(* 5 safety invariants must be formally proved before any hardware deployment *)

(* I1: No two vehicles simultaneously hold passage rights in the same zone *)

NoConflict == \A z \in Zones, v1, v2 \in Vehicles :

(v1 # v2 /\ InSlot(v1,z) /\ InSlot(v2,z))

=> SlotStart(v1,z) >= SlotEnd(v2,z) \/ SlotStart(v2,z) >= SlotEnd(v1,z)

(* I2: Every ConsensusToken expires exactly 300ms after issue *)

TokenExpiry == \A t \in Tokens : t.expiry = t.issued + 300

(* I3: No constraint output in observe-only mode *)

SafeObserve == \A v \in Vehicles : v.mode = OBSERVE => v.sco_type = NONE

(* I4: Negotiation always terminates (no deadlock) *)

NoDeadlock == \A s \in Sessions : <>[](s.state \in {COMPLETE, FALLBACK})

(* I5: Emergency vehicles always receive first passage slot *)

EmergencyFirst == \A t \in Tokens :

(\E v \in t.participants : v.priority = EMERGENCY)

=> t.sequence[0] \in EmergencyVehicles

# SECTION 11 — Build Roadmap & Phase Timeline

The following Gantt-style timeline shows all six build phases across 18 months to V1 launch. Phases overlap where earlier phases produce components required by later phases. Milestones (★) mark the point at which a phase's primary deliverable is complete and the next phase is unblocked.

| BUILD ROADMAP — PHASE TIMELINE (18 MONTHS TO V1 LAUNCH) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PHASE | M1 | M2 ★ | M3 | M4 | M5 ★ | M6 | M7 | M8 ★ | M9 | M10 | M11 ★ | M12 | M13 ★ | M14 | M15 | M16 ★ | M17 | M18 ★ |
| Phase 0: Foundations Environment, toolchain, formal spec | ▶ | ■ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase 1: Core Protocol J2735 extensions, FSM, DCC, geohash |  | ▶ | ━ | ━ | ■ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase 2: Safety & Security ASIL-D partition, Trust Score, HSM |  |  |  |  | ▶ | ━ | ━ | ■ |  |  |  |  |  |  |  |  |  |  |
| Phase 3: AI Integration LSTM predictor, perception adapter |  |  |  |  |  |  |  | ▶ | ━ | ━ | ■ |  |  |  |  |  |  |  |
| Phase 4: HIL Testing Real hardware, latency validation |  |  |  |  |  |  |  |  |  |  | ▶ | ━ | ■ |  |  |  |  |  |
| Phase 5: Shadow Mode Field Trial 100k+ intersection events |  |  |  |  |  |  |  |  |  |  |  |  | ▶ | ━ | ━ | ■ |  |  |
| Phase 6: Active Trial Regulatory approval, V1 launch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ▶ | ━ | ■ |
| ★ MILESTONES |  | ODD Frozen |  |  | Protocol Sim Pass |  |  | ASIL-D Cert Ready |  |  | AI >87% Accuracy |  | HIL Latency Pass |  |  | 100k Events |  | V1 Launch |

Figure 11 — Build Phase Timeline: 18 months from foundations to V1 active deployment, with 7 key milestones and phase overlaps shown

### 11.1 Phase Gate Criteria

| Phase | Exit Gate | Blocking Dependencies | Primary Deliverable |
| --- | --- | --- | --- |
| Phase 0 | ODD frozen; TLA+ spec written; simulation environment running | None | Formal specification document + sim environment |
| Phase 1 | Protocol sim test suite 100% pass; zero passage conflicts in 10k runs | Phase 0 complete | Working QM protocol stack with simulation validation |
| Phase 2 | All adversarial tests pass; HSM <3ms verified; ASIL-D partition running | Phase 1 complete | Production-ready security + safety partition architecture |
| Phase 3 | AI model >87% accuracy; <8ms inference on target NPU; all V1 FSMs complete | Phase 2 complete | Full Handshake V1 software stack |
| Phase 4 | All HIL latency targets met; fault injection 100% safe-state; ECU certified | Phase 3 complete | Certified hardware deployment package |
| Phase 5 | 100,000 shadow mode events; safety case evidence package assembled | Phase 4 complete | Regulatory submission package |
| Phase 6 | Road authority TRP approval; active trial clearance; V1 metrics baseline set | Phase 5 complete | V1 live deployment |

# SECTION 12 — ISO 26262 Compliance

### 12.1 ASIL Decomposition

Handshake's safety-critical constraint output function targets ASIL-D. To achieve this within a tractable development effort, ASIL-D is decomposed into two independent ASIL-B channels per ISO 26262 Part 9 — the main Rule Engine and a redundant Safety Monitor that independently validates all outputs.

| ISO 26262 Attribute | Value | Evidence |
| --- | --- | --- |
| Item Type | Driver Assistance / Cooperative Safety | System design document |
| Worst-Case Hazard | Misleading right-of-way causes intersection collision | HARA completed |
| Severity (S) | S3 — Life-threatening | HARA worksheet |
| Exposure (E) | E4 — Daily urban driving | Field statistics |
| Controllability (C) | C2 — Normally controllable (driver can override) | Architecture review |
| Resulting ASIL | ASIL-D | HARA outcome |
| Decomposition | ASIL-B (Rule Engine) + ASIL-B (Safety Monitor) | ISO 26262-9 Clause 5 |
| Software Dev Standard | MISRA-C++ 2023 | Code review toolchain |
| Formal Verification | TLA+ / TLC model checking | Verification report |
| Memory Model | Static allocation only — no heap | MISRA-C++ Rule 18-4-1 |

### 12.2 ISO 26262 Clause Coverage

| Clause | Part | Handshake Evidence |
| --- | --- | --- |
| Freedom from interference | P6 §6 | Hypervisor QM/ASIL-D partition (Section 2 + Figure 2) |
| Software unit design | P6 §8 | MISRA-C++ FSM; static allocation; no exceptions |
| Formal verification | P6 §9 | TLA+ invariants (Section 10.1 + Figure FSM diagram) |
| HW-SW interface | P4 §7 | SafetyEnvelope 128-byte fixed interface (Section 5 + Figure 6) |
| Safe state definition | P4 §8 | Observe-only + yield-all, formally defined (Section 6) |
| Diagnostic coverage | P5 §8 | Watchdog + safety counters + ADAS heartbeat |
| Dependent failures | P9 §7 | Ring buffer architecture: QM failure cannot enter ASIL-D |
| SW verification | P6 §11 | MC/DC testing + adversarial injection + HIL validation |
| Calibration data | P8 §9 | TRP files: signed by road authority, version-controlled |
| Configuration management | P2 §7 | Git-based CM with ASIL-D traceability matrix |

# SECTION 13 — Compute & Platform Specifications

### 13.1 Resource Budget (NVIDIA Drive Orin Reference Platform)

| Subsystem | Partition | CPU Cores | NPU/GPU | RAM | Notes |
| --- | --- | --- | --- | --- | --- |
| C-V2X Stack & DCC | QM | 1 A55 pinned | None | 64 MB | Dedicated core prevents scheduler interference |
| J2735 Parser + SCMS | QM | 1 A55 pinned | None | 32 MB | All crypto offloaded to HSM |
| AI Intent Predictor | QM | 0.3 avg share | 15 TOPS burst | 128 MB | INT8; burst during negotiation window only |
| Perception Adapter | QM | 0.2 avg share | Existing ADAS | 32 MB | Consumes fused object list only |
| Handshake Core FSM | ASIL-D | 1 ASIL-D core | None | 4 MB | Static allocation; no heap; dedicated core |
| Safety Monitor | ASIL-D | 0.1 share | None | 1 MB | Independent time partition; watchdog |
| TOTAL HANDSHAKE |  | ~2.6 cores | 15 TOPS burst | 261 MB | Leaves >60% Drive Orin headroom for ADAS |

### 13.2 Supported SoC Platforms

| Platform | Hypervisor | HSM | Target | Status |
| --- | --- | --- | --- | --- |
| NVIDIA Drive Orin (254 TOPS) | Green Hills INTEGRITY / QNX | Integrated | Primary | Fully validated |
| Qualcomm Snapdragon Ride Elite | QNX Hypervisor 8.0 | Integrated | Secondary | Full support |
| NXP S32G3 | PikeOS / Green Hills | Integrated | Secondary | Full support |
| Renesas R-Car V4H | Proprietary in development | Integrated | Conditional | Hypervisor pending |
| Mobileye EyeQ Ultra | Proprietary OS | Available | API-level only | SCO interface only |

# SECTION 14 — Complete Feature Specification

### 14.1 Core V2V Communication

- Automatic peer discovery and session initiation within 100ms of first beacon receipt.

- Multi-vehicle negotiation groups: 2–12 active negotiators per conflict zone in V1.

- Decentralized mesh — no cloud, no central controller required.

- Offline-first architecture: full V2V operation via C-V2X PC5 direct sidelink.

- Dynamic session lifecycle: sessions auto-create on conflict detection, auto-terminate on zone exit.

### 14.2 Negotiation & Right-of-Way

- Bilateral NegotiationRequest / NegotiationGrant / ConsensusToken protocol — not broadcast-only.

- Hard 300ms ConsensusToken validity with GNSS-synchronized expiry.

- 5-step deterministic tie-breaking (emergency priority → bus priority → traffic law → ETA → pseudonym hash).

- RSU arbitration fallback for failed peer-to-peer negotiations.

- V1 behavior library: 4-way stop | intersection yield | low-speed merge | emergency preemption.

### 14.3 AI & Intelligence

- LSTM intent predictor: 5-class maneuver classification at 1.5s horizon, >87% accuracy.

- Full legacy vehicle support: sensor-only inference for non-Handshake vehicles.

- Collision Risk Index: continuous probabilistic estimate per vehicle pair in geohash cell.

- Behavioral anomaly detection: trajectory consistency monitoring with trust score feedback.

### 14.4 Security

- SCMS pseudonym certificates with 10-minute rotation cycle.

- HSM-accelerated ECDSA: signature verification <3ms, signing <2ms.

- Bloom Filter CRL propagation: offline revocation information without cellular dependency.

- 4-stream Trust Score Engine: continuous per-sender graded confidence.

- Replay prevention: dual mechanism (timestamp + sequence number).

- Sybil attack detection: position-clustering analysis across multiple IDs.

### 14.5 Infrastructure

- RSU software upgrade (no hardware replacement) enables authoritative arbitration.

- Traffic Rule Profile (TRP) system: jurisdiction-specific, authority-signed rule files.

- TMC OpenAPI interface: anonymized analytics, incident alerts, TRP distribution.

- OTA model and TRP updates via signed package delivery.

### 14.6 System Management

- ODD continuous enforcement: instant observe-only transition on any boundary violation.

- Three-tier graceful degradation: Active → Observe-Only → Shutdown.

- ASIL-D watchdog with independent time partition and ADAS heartbeat.

- Self-test on startup and every 60 seconds: C-V2X radio, GNSS, HSM, ASIL-D health.

- Privacy-safe event logging with configurable TMC upload for safety analytics.

# SECTION 15 — Technology Stack

| Layer | Technology | Purpose | COTS / Custom |
| --- | --- | --- | --- |
| Communication | C-V2X PC5 (3GPP Rel.14+) | Direct V2V radio | COTS — unchanged |
| Message Protocol | SAE J2735 + 3 Handshake extensions | Packet encoding | COTS base + Custom extensions |
| Congestion Control | ETSI TS 102 687 DCC | Channel management | COTS algorithm + Custom thresholds |
| Security PKI | SCMS (CAMP / DigiCert) | Certificate management | COTS + config change |
| Crypto Acceleration | ECDSA P-256 via HSM HAL | Message signing | COTS HSM + Custom HAL |
| Offline Revocation | Bloom Filter over V2V BEACON | Offline CRL propagation | Custom (novel) |
| Trust Scoring | 4-stream Trust Score Engine | Graded confidence | Custom (novel) |
| Safety Isolation | Automotive hypervisor | ISO 26262 partitioning | COTS hypervisor + custom config |
| Safety Core | ASIL-D MISRA-C++ FSM | Right-of-way decisions | Custom (core IP) |
| AI Inference | LSTM+Conv1D INT8 (ONNX/TFLite) | Intent prediction | Custom model + COTS runtime |
| Geospatial | Geohash precision-7 | Negotiation group bounding | COTS algorithm |
| Simulation | SUMO + CARLA + Veins | Validation environment | COTS + Custom modules |
| Formal Verification | TLA+ / TLC | FSM correctness proofs | COTS tool + Custom specs |
| Infrastructure API | OpenAPI 3.0 REST | TMC interface | Custom API |

HANDSHAKE  —  Master Technical Blueprint v4.1  (Illustrated Edition)

11 technical diagrams  •  15 specification sections  •  Full ISO 26262 ASIL-D compliance path  •  Build-ready engineering blueprint