HANDSHAKE V6  ·  SIMULATION TECHNICAL SUMMARY

Python  ·  Two-VM Network  ·  26 Automated Tests  ·  Full Protocol Implementation

What Handshake V6 Is

Handshake V6 is a Python-based discrete-event simulation of the Handshake Negotiation Protocol running across two virtual machines connected over a real UDP multicast network. It is not a visualisation or a mockup. It is a running implementation of the HNP protocol state machine, exercised under realistic conditions — packet loss, multi-vehicle contention, emergency preemption, weather degradation, and adversarial rogue behaviour — with every interaction logged, tested, and verifiable.

The simulation was built to validate the protocol design before formal verification and hardware implementation. Its 26 automated tests function as a protocol regression suite: if any change to the core FSM, token logic, or safety rules breaks a behavioural guarantee, a test fails. This makes V6 both a proof of concept and a living specification.

Architecture Overview

| Two-Simulator Design V6 contains two independent simulators that run in parallel and share a V2X radio layer: Road Simulator (sim_road.py) Multi-lane highway environment. Cars spawn, drive, change lanes, platoon, and complete journeys. Models the token-based communication protocol between moving vehicles at speed. Intersection Simulator (sim_intersection.py) Four-arm signalised intersection. Cars queue on all four arms, negotiate right-of-way using the HNP token protocol, cross the intersection box, and exit. This is the primary HNP validation environment. | 27 Python Modules Module; Role. sim_road / sim_intersection; Core simulators. car_road / car_intersection; Vehicle state machines. arm_queue.py; Intersection queue manager. tokens.py; Token lifecycle management. traffic_light.py; Signal phase controller. radio.py / radio_network.py; V2X UDP multicast layer. dashboard_road / _intersection; Rich terminal dashboards. law.py; CVC traffic law rules. ml_predictor.py; AI prediction interface. behavior_profiles.py; Driver behaviour profiles. sim_logger.py; Structured logging + EDR. config.py; Protocol constants + enums. |
| --- | --- |

Vehicle Classes and Protocol Behaviour

| Class | Token Protocol | Special Behaviour | Dashboard Glyph | Lane Change |
| --- | --- | --- | --- | --- |
| SMART | Full HNP — NEG_REQUEST → TOKEN_ISSUED → TOKEN_ACK → CROSSING | V2X radio, ML prediction, platoon capable | ► bright yellow | Smooth, gap-aware |
| LEGACY | No V2X — yields on green signal only | Human-driver behaviour profiles (6 types) | ► white | Indicator-based |
| ROGUE | Sends NEG_REQUEST, ignores yields | May run red, contest illegally — triggers ROGUE_VIOLATION | ► bold magenta | Aggressive |
| EMERGENCY | Full HNP + PREEMPT broadcast | Forces AMBULANCE_CORRIDOR — all others yield, lights preempted | 🚨 red beacon | Shoulder-capable |
| PLATOON | Coordinated convoy protocol | CACC gap control, platoon join/leave/dissolve FSM | ► bright magenta | Formation-locked |

The Two-VM Setup

V6 runs on two VirtualBox virtual machines connected over a Host-Only network. Each VM runs a separate instance of the simulation. The radio layer (radio_network.py) uses UDP multicast (group 239.0.0.4:5400) to simulate C-V2X PC5 sidelink communication between the two machines. This tests the protocol under real network conditions — actual UDP packet scheduling, OS-level jitter, and interface binding — not simulated delays.

| VM 1 — Machine 1 (machine_id=1) - Runs Road Simulator with Player 1 (P1) vehicle - Runs Intersection Simulator — North and West arms - Initiates token negotiations as machine 1 - Sends/receives V2X messages over multicast | VM 2 — Machine 2 (machine_id=2) - Runs Road Simulator with Player 2 (P2) vehicle - Runs Intersection Simulator — South and East arms - Participates in token sessions initiated by either VM - Token holder shown in green ★ on both dashboards simultaneously |
| --- | --- |

Dashboard

V6 includes two real-time terminal dashboards built with the Rich library, refreshing at 5Hz. The intersection dashboard renders a fixed ASCII crossroads with colour-coded directional arrows (↓↑→← pointing toward centre), live queue positions on all four arms, crossing cars with progress bars, token exchange log, event log, and a live stats panel. The road dashboard renders a per-lane heatmap, all vehicle positions with speed and state, weather indicator, and token event stream.

Test Coverage — 26 Automated Tests

| # | Test Category | What It Verifies | Protocol Invariant Covered |
| --- | --- | --- | --- |
| 1–4 | Token negotiation cycle | NEG_REQUEST → TOKEN_ISSUED → TOKEN_ACK → CROSSING → CROSSED completes for all car types | NegotiationTermination |
| 5–6 | Emergency corridor | PREEMPT broadcast clears all arms; corridor forms within 3 ticks; normal operation resumes after passage | EmergencyPriority |
| 7–8 | Rogue vehicle handling | ROGUE_VIOLATION logged on red-light violation; rogue does not receive token while in violation | NoConflictingPassage |
| 9–10 | Pedestrian phase | ALL_RED triggered on pedestrian crossing; no car receives TOKEN_ISSUED during pedestrian phase | NoConstraintInObserveMode |
| 11–13 | Weather degradation | All 6 weather states propagate to all cars within 1 tick; ICE state reduces speeds to ≤50% of nominal | Weather safety |
| 14–16 | Platoon management | Platoon forms, maintains gap, and dissolves cleanly; no collisions across 3×30s runs | NoConflictingPassage |
| 17–18 | Multi-arm queue ordering | Closest cars always display adjacent to box on W and E arms; queue advances after crossing | Protocol correctness |
| 19–20 | Status field contracts | All fields read by dashboards verified present in sim_road and sim_intersection status dicts | Interface stability |
| 21–22 | Dashboard render | render_road() and render() produce valid Rich layout objects without exception | Dashboard integrity |
| 23–24 | Road sim zero collisions | Road sim runs 30s with full NPC mix including platoon; collision_count = 0 | NoConflictingPassage |
| 25–26 | Import and syntax | All 27 modules import cleanly; all 27 files parse without SyntaxError | Codebase integrity |

What V6 Demonstrates and What It Does Not

| What V6 DEMONSTRATES | What V6 Does NOT Claim |
| --- | --- |
| - The HNP protocol FSM is implementable and terminates correctly - Token negotiation produces consistent passage ordering across all vehicle types - Emergency preemption and corridor formation work reliably - Weather degradation propagates correctly and affects vehicle behaviour - The ASIL-D/QM architectural separation is correctly designed - The protocol handles adversarial (rogue) vehicles without unsafe state - Two-machine UDP multicast correctly simulates C-V2X PC5 radio characteristics | - V6 is NOT a formal proof — TLA+/TLC verification is Phase 1 of the PhD programme - V6 does NOT run on C-V2X silicon — hardware validation is Phase 4 - V6 does NOT implement the HSM cryptographic pipeline - V6 does NOT implement the ML models to production accuracy - V6 does NOT simulate multi-vehicle radio channel contention at full scale |

V6 is the proof of concept. The PhD programme — or an industrial co-development partnership — is the mechanism by which it becomes a formally verified, hardware-validated, deployable protocol.

Baah Gabriel   ·   tebrigabby@gmail.com   ·   +233 595 325 487   ·   Ghana   ·   March 2026