HANDSHAKE V6

Traffic Simulation — Documentation

Setup · Objects · Commands · Multi-PC

# 1  Overview

Handshake V6 is a Python traffic simulation that runs two independent scenarios: a road simulation and a four-arm intersection simulation. Both use a V2X radio protocol (Vehicle-to-Everything) where smart cars negotiate passage by broadcasting messages and receiving tokens. Legacy and rogue cars test the robustness of the protocol. An ML layer learns from each run and begins predicting congestion and anomalies over time.

## 1.1  Two Simulations

| File | What it simulates |
| --- | --- |
| road.py | 5 km single-direction road with 3 lanes. Smart NPCs, legacy cars, player car, emergency vehicle, school zones, roadworks zone, weather, fuel, heatmap. |
| intersection.py | 4-arm traffic intersection (North/East/South/West). Token-based passage protocol, traffic light, pedestrians, ambulance corridor, rogue cars, ML predictions. |
| main_road.py | Advanced road entry point with extra flags: rush hour, breakdown, platoon, convoy, roadworks, split-brain. |
| main_intersection.py | Advanced intersection entry point with extra flags: rush hour, rogue, ambulance, pedestrian, machine fail, SPaT delay, multi-PC mode. |

Both simulations run at 10 Hz (one tick every 0.1 s). The dashboard refreshes in the terminal in real time using the Rich library.

# 2  Objects Reference

## 2.1  Road Cars

All road cars inherit from BaseRoadCar. Each car has a position on the road, a lane, a speed, a driving state, a fuel gauge, and a reaction-time buffer.

| Class | Type | Behaviour |
| --- | --- | --- |
| SmartNPC | SMART | Has V2X radio. Broadcasts BEACON, sends INTENT messages, uses GRU ML predictor for gap anticipation. Obeys all CVC rules. Follows driver profile. |
| LegacyNPC | LEGACY | No radio. Reacts only to physical gaps ahead. Follows driver profile. Can trigger near-misses. |
| EmergencyNPC | EMERGENCY | Broadcasts EMERG_PREEMPT. All cars yield and move to shoulder (CVC 21806). Travels at 80 km/h, ignores speed zones. |
| BreakdownNPC | BREAKDOWN | Starts broken down in the breakdown zone. Broadcasts HAZARD. Cars approaching within 300 m slow and move to lane 0. |
| PlatoonNPC | PLATOON | Travels in a tight convoy at 3.5 m spacing. Leader controls speed; followers match via radio. Enabled with --platoon. |
| ConvoyNPC | SMART | Large vehicles in convoy formation. 12 m spacing. Enabled with --convoy. |
| PlayerCar | SMART | Controlled by the user or autopilot. Has full V2X capability. Receives ML predictions. Displayed in the dashboard. |

## 2.2  Intersection Cars

Intersection cars queue in one of four arms (North, East, South, West) and negotiate crossing via the token protocol.

| Class | Type | Behaviour |
| --- | --- | --- |
| SmartCar | SMART | Sends NEG_REQUEST during the negotiation window. Waits for PASSAGE_TOKEN. Departs in assigned slot. Runs M3 anomaly scoring each tick. |
| LegacyCar | LEGACY | No radio. Creeps forward when green and the gap ahead is safe. No token participation. |
| EmergencyCar | EMERGENCY | Broadcasts EMERG_PREEMPT. Gets Slot 0 immediately. Zero startup delay. If a token is already live, it is cancelled and reissued. |
| RogueCar | ROGUE | Has radio and receives the token but ignores its slot. Crosses out of order. Logged as ROGUE_VIOLATION. M3 anomaly scorer will flag it. |

## 2.3  Driver Profiles

Every NPC gets one profile at spawn. Profiles control reaction time, following distance, braking aggressiveness, and lane-change tendency. Smart cars still obey CVC rules; profiles adjust thresholds, not the law gate.

| Profile | Reaction | Character |
| --- | --- | --- |
| COMMUTER | 0.8 s | Steady, predictable. Keeps 3 s headway. Most common (30%). |
| TAILGATER | 0.5 s | Closes gaps aggressively. High near-miss rate. 1 s headway target. |
| HESITANT | 1.3 s | Large gaps, slow to accelerate, rarely overtakes. Can cause bunching. |
| LATE_BRAKER | 1.5 s | Slowest reaction. Brakes hard at the last moment. Common near-miss trigger. |
| AGGR_OT | 0.6 s | Frequent overtakes, high speed variance. Can violate CVC 21750. |
| WANDERER | 1.2 s | Inconsistent — randomly drifts between lanes. Distraction model active. |

## 2.4  Road Zones (5 km road)

| Zone | Position | Speed limit / rule |
| --- | --- | --- |
| Entry | 0 – 500 m | 48 km/h (default road speed) |
| 🏫 School Zone 1 | 600 – 900 m | 24 km/h — CVC 22352a. All cars slow. Logged if violated. |
| Open Road 1 | 900 – 1800 m | 48 km/h |
| 🔶 Breakdown Zone | 1200 – 1600 m | Hazard if BreakdownNPC present. 300 m pre-warn distance. |
| 🚧 Roadworks | 2000 – 2400 m | 16 km/h — lane reduced to 1. CVC 22352. |
| Highway | 2400 – 3800 m | 48 km/h |
| 🏫 School Zone 2 | 3900 – 4200 m | 24 km/h — CVC 22352a. |
| Exit | 4200 – 5000 m | 48 km/h |

## 2.5  Intersection Token Protocol

The intersection uses a handshake protocol to coordinate which cars cross the box. There are four stages per green phase:

- Negotiation window opens (0.8 s). Smart cars send NEG_REQUEST with their queue position and car type.

- TokenManager collects requests, resolves conflicts, and assigns ordered slots.

- PASSAGE_TOKEN broadcast: each car gets a slot number (0 = first to go).

- Cars depart in slot order, 1.2 s apart. Slot gap adjusts if the ML clearance predictor is confident.

Emergency cars skip straight to Slot 0 and preempt the light. Rogue cars receive the token but ignore their slot.

## 2.6  Weather System

Weather changes every 60 seconds on the road simulation. Each state affects speed limit, following distance, braking power, and visibility range.

| State | Speed | Brake power | Follow gap | Description |
| --- | --- | --- | --- | --- |
| ☀ Clear | 100% | 100% | ×1.0 | Normal conditions |
| 🌧 Light Rain | 90% | 85% | ×1.3 | Stopping dist +30% |
| ⛈ Heavy Rain | 75% | 70% | ×1.6 | Speed -25%, braking -30% |
| 🌫 Fog | 65% | 90% | ×1.5 | Visibility 30 m, -35% speed |
| ❄ Light Snow | 70% | 65% | ×1.7 | Speed -30%, braking -35% |
| 🧊 Ice | 50% | 40% | ×2.2 | DANGER — braking power -60% |

## 2.7  Machine Learning Layer

The ML layer runs in the background and improves across multiple simulation runs. No data is sent externally — all training is local.

| Feature | Model | Effect |
| --- | --- | --- |
| M1 | Intersection data extractor | After each intersection run, extracts per-arm tick-level features and saves NPZ training files. |
| M2 | Queue clearance GRU (48→24 hidden) | Predicts how long an arm's queue will take to clear. When confident (≥0.6), the TokenManager tightens or widens the platoon slot gap automatically. |
| M3 | Anomaly autoencoder (180→32→180) | Trained on normal behaviour only. Each SmartCar scores its own 30-tick feature window. Score > 0.7 with no token → ROGUE_ALERT broadcast. |
| Road | Road GRU predictor (64→32 hidden) | Predicts position, speed, and next action 2 s ahead. SmartNPCs use it to anticipate gaps. Confidence shown in dashboard. |

Models are stored in the model/ folder and loaded automatically on the next run. Cold start (no model file) is safe — confidence is 0.0 and the sim falls back to reactive driving.

## 2.8  Radio / Messages

All V2X communication goes through the Radio class. Single-machine runs use in-process queues. Multi-machine runs use UDP multicast on 239.0.0.4:5400. Packet loss is configurable with --loss.

| Message type | Meaning |
| --- | --- |
| BEACON | Broadcast every tick: position, speed, state, lane. |
| INTENT_OVERTAKE | Signals lane change intention 100 m ahead (CVC 22107). |
| HARD_BRAKE | Warns following cars of emergency braking. |
| EMERG_PREEMPT | Emergency vehicle clears the road (CVC 21806). |
| HAZARD | Breakdown car warning — slow down and move right. |
| NEG_REQUEST | Intersection — car requests a crossing slot. |
| PASSAGE_TOKEN | Intersection — TokenManager grants ordered slots. |
| TOKEN_CANCEL | Active token revoked (emergency preemption). |
| ROGUE_CROSS | Rogue car announces it is ignoring its slot. |
| SPAT | Signal Phase and Timing broadcast from traffic light. |
| ROGUE_ALERT | M3 anomaly alert — car moving without token detected. |

## 2.9  CVC Law Codes

The simulation models California Vehicle Code (CVC) rules. Violations are logged to logs/VIOLATIONS.txt.

| Code | Rule |
| --- | --- |
| 21450 | Green signal — may proceed |
| 21453 | Red/yellow signal — must stop |
| 21703 | Safe following distance |
| 21750 | Overtake left only, only when safe |
| 21806 | All vehicles yield to emergency and pull right |
| 21800 | Right of way at uncontrolled intersection (--uncontrolled mode) |
| 21950 | Yield to pedestrian in crosswalk |
| 22107 | Signal 100 ft before lane change |
| 22352a | School zone — 15 mph (24 km/h) |
| 22500 | No stopping in hazardous position |

## 2.10  Log Files

All log files are written to the logs/ folder automatically after each run.

| File | Contents |
| --- | --- |
| logs/HISTORY.txt | One line per run: timestamp, type, cars, events. |
| logs/VIOLATIONS.txt | Cumulative CVC violation ledger across all runs. |
| logs/<type>_<ts>.json | Full run data: events, snapshots, car trajectories (used for ML training). |
| logs/<type>_<ts>.txt | Human-readable run summary. |
| logs/training/*.npz | Extracted feature arrays used to train the ML models. |
| model/predictor.pt | Road GRU model weights (loaded at start if present). |
| model/int_predictor.npz | Intersection clearance GRU model weights. |
| model/anomaly_scorer.npz | Anomaly autoencoder weights. |

# 3  Environment Setup

## 3.1  Requirements

The simulation requires Python 3.10 or newer and two Python packages:

Python   3.10+

numpy    (matrix operations, ML models)

rich     (terminal dashboard)

No other external packages are needed. The ML layer uses only numpy — no PyTorch or TensorFlow required.

## 3.2  macOS

- Install Python 3 (skip if already installed):

brew install python3

# or download from python.org/downloads

- Go to the simulation folder:

cd path/to/handshake_v6_final

- Install dependencies:

pip3 install numpy rich

- Run:

python3 road.py

python3 intersection.py

The terminal must be at least 120 columns wide for the dashboard to display correctly. To check:

tput cols

## 3.3  Linux (Ubuntu / Debian)

- Install Python 3 and pip:

sudo apt update

sudo apt install python3 python3-pip

- Go to the simulation folder:

cd path/to/handshake_v6_final

- Install dependencies:

pip3 install numpy rich

- Run:

python3 road.py

python3 intersection.py

On headless servers (no terminal): use --no-dashboard to get plain text output.

## 3.4  Windows

- Download Python 3 from python.org/downloads. During install, check 'Add Python to PATH'.

- Open Command Prompt or PowerShell. Go to the simulation folder:

cd path\to\handshake_v6_final

- Install dependencies:

pip install numpy rich

- Run:

python road.py

python intersection.py

Windows Terminal is recommended. The built-in cmd.exe may not render emoji correctly. If the dashboard looks broken, use Windows Terminal (free from the Microsoft Store) or add --no-dashboard.

## 3.5  Quick Test (all platforms)

Run a 5-second headless test to confirm everything is working:

# Road simulation

python3 road.py --no-dashboard --runtime 5

# Intersection simulation

python3 intersection.py --no-dashboard --runtime 5

Both should print a summary and exit cleanly with no errors.

# 4  Commands

## 4.1  Road Simulation  (road.py / main_road.py)

### Basic usage

python3 road.py                          # standard run, with dashboard

python3 road.py --no-dashboard           # plain text output

python3 road.py --runtime 120            # stop after 120 seconds

python3 road.py --spectator              # watch only, no player car

### Traffic scenarios

python3 main_road.py --rush              # rush hour — more cars on road

python3 main_road.py --breakdown         # add a broken-down car

python3 main_road.py --platoon           # add a platooning group

python3 main_road.py --convoy            # add a truck convoy

python3 main_road.py --roadworks         # enable roadworks zone

### Network / fault injection

python3 main_road.py --loss 0.1          # 10% random packet loss

python3 main_road.py --loss 0.3          # 30% packet loss

python3 main_road.py --split-brain       # simulate network partition

python3 main_road.py --spat-delay 2.0    # inject 2s SPaT signal delay

### NPC count

python3 main_road.py --smart 6 --legacy 2 --emerg 1

# --smart N    number of SmartNPC cars (default 4)

# --legacy N   number of LegacyNPC cars (default 4)

# --emerg N    number of EmergencyNPC cars (default 1)

### ML

python3 main_road.py --no-learn          # skip ML training after run

## 4.2  Intersection Simulation  (main_intersection.py)

### Basic usage

python3 main_intersection.py                         # standard run

python3 main_intersection.py --no-dashboard          # plain text

python3 main_intersection.py --cars 10               # 10 cars per arm

python3 main_intersection.py --runtime 120           # stop after 120 s

python3 main_intersection.py --no-log                # skip writing log files

### Traffic scenarios

python3 main_intersection.py --rush-hour

# 40 cars per arm, auto-triggers ambulance corridor at T+40 s

python3 main_intersection.py --rogue

# One rogue car per arm ignores its token slot

python3 main_intersection.py --ambulance

# Triggers ambulance corridor at T+15 s  (CVC 21806)

python3 main_intersection.py --pedestrian

# Triggers pedestrian crossing at T+10 s  (CVC 21950)

python3 main_intersection.py --uncontrolled

# No traffic light — right-of-way negotiation  (CVC 21800)

### Fault injection

python3 main_intersection.py --machine-fail 2

# Arm 2 (East) goes offline at T+25 s, recovers at T+45 s

python3 main_intersection.py --spat-delay 1.5

# Inject 1.5 s SPaT signal delay — stale phase events logged

python3 main_intersection.py --loss 0.2

# 20% packet loss on all V2X messages

### Combine flags

python3 main_intersection.py --rush-hour --rogue --ambulance

python3 main_intersection.py --cars 30 --loss 0.1 --spat-delay 0.5

# 5  Multi-PC Setup

Both simulations can be split across multiple machines on the same network. Machines communicate via UDP multicast on 239.0.0.4:5400. All machines must be on the same LAN (or VLAN).

## 5.1  Network Requirements

- All machines on the same LAN or same subnet.

- UDP multicast must not be blocked by the router or firewall.

- Open UDP port 5400 on every machine.

- Python code folder must be identical on every machine (copy it over or use a shared drive).

Firewall commands:

# macOS — no extra steps needed (multicast is allowed by default)

# Linux (Ubuntu/Debian)

sudo ufw allow 5400/udp

# Windows — run in PowerShell as Administrator

New-NetFirewallRule -DisplayName "Handshake V6" -Direction Inbound -Protocol UDP -LocalPort 5400 -Action Allow

## 5.2  Road Simulation — 2 Machines

The 5 km road is split in half: Machine 1 owns km 0–2.5, Machine 2 owns km 2.5–5.0. Start Machine 1 first.

# Machine 1  (first half of road)

python3 main_road.py --vms 2 --machine 1

# Machine 2  (second half of road, start within 5 seconds)

python3 main_road.py --vms 2 --machine 2

Each machine shows its own segment in the dashboard. Cars that cross the midpoint are handed off automatically.

## 5.3  Intersection Simulation — 2 Machines

Machine 1 controls North + South arms and hosts the traffic light. Machine 2 controls East + West arms and mirrors the light phase via SPaT.

# Machine 1  (North + South + traffic light host) — start first

python3 main_intersection.py --machine 1 --vms 2

# Machine 2  (East + West) — start within 5 seconds

python3 main_intersection.py --machine 2 --vms 2

## 5.4  Intersection Simulation — 4 Machines (1 arm each)

Each machine controls one arm. Machine 1 also hosts the traffic light. Start Machine 1 first, then the others in any order.

# Machine 1 — North arm + traffic light host — start first

python3 main_intersection.py --machine 1

# Machine 2 — East arm

python3 main_intersection.py --machine 2

# Machine 3 — South arm

python3 main_intersection.py --machine 3

# Machine 4 — West arm

python3 main_intersection.py --machine 4

## 5.5  Multi-PC Troubleshooting

| Symptom | Fix |
| --- | --- |
| Machines don't see each other | Check that UDP 5400 is open on both machines. Try disabling the firewall temporarily. |
| Multicast not working on Linux | Run: sudo route add -net 224.0.0.0 netmask 240.0.0.0 dev eth0 (replace eth0 with your interface) |
| Cars not appearing on Machine 2 | Make sure Machine 1 started first and is running. Both must use the same --vms value. |
| High packet loss seen in dashboard | Check network congestion or whether the router blocks multicast. Switch to wired Ethernet. |
| SPaT stale events on Machine 2 | Normal if network latency > 0.4 s (SPaT interval). Use --spat-delay 0 to suppress logging. |

# 6  Key Configuration Values

All constants live in config.py. Edit them to change simulation behaviour.

| Constant | Default | What it does |
| --- | --- | --- |
| CARS_PER_ARM | 20 | Cars per intersection arm in normal mode. |
| RUSH_HOUR_CARS | 40 | Cars per arm in rush-hour mode. |
| GREEN_S | 25.0 s | Green phase duration. |
| PLATOON_MAX | 10 | Max cars in one intersection platoon (token). |
| PLATOON_GAP_S | 1.2 s | Time gap between consecutive slot departures. |
| SPEED_LIMIT_MS | 13.4 m/s | Default road speed (~48 km/h). |
| FOLLOW_TIME_S | 3.0 s | Default headway in seconds (safe following distance). |
| FUEL_RANGE_M | 8000 m | Full tank range. At 10% remaining, car slows to 40 km/h. |
| NEAR_MISS_M | 1.5 m | Gap below this triggers a NEAR_MISS event. |
| HEATMAP_BUCKETS | 20 | Number of road speed heatmap segments. |
| REACTION_BUFFER_TICKS | 15 | Reaction time buffer depth (15 ticks = 1.5 s max). |
| WEATHER_CHANGE_INTERVAL_S | 60 s | How often the weather state changes. |
| NEG_WINDOW_S | 0.8 s | Intersection negotiation window length. |
| TOKEN_TIMEOUT_S | 5.0 s | Token fallback timer if acknowledgement is lost. |
| OUTBOUND_HOLD_TICKS | 5 | Ticks a crossed car stays visible in the outbound lane. |
| INT_STARTUP_PER_POS | 0.3 s | Extra startup delay per queue position (realism). |
| MCAST_GRP / MCAST_PORT | 239.0.0.4 / 5400 | Multicast address and port for multi-machine mode. |

# 7  Quick Reference

## 7.1  Common command examples

# Road — standard

python3 road.py

# Road — headless test

python3 road.py --no-dashboard --runtime 10

# Intersection — with rogue cars and ambulance

python3 main_intersection.py --rogue --ambulance

# Intersection — rush hour, no logging

python3 main_intersection.py --rush-hour --no-log

# Intersection — uncontrolled (no traffic light)

python3 main_intersection.py --uncontrolled

# Intersection — 4 machines (run each on a separate PC)

python3 main_intersection.py --machine 1    # PC 1 — start first

python3 main_intersection.py --machine 2    # PC 2

python3 main_intersection.py --machine 3    # PC 3

python3 main_intersection.py --machine 4    # PC 4

# Road — 2 machines

python3 main_road.py --vms 2 --machine 1    # PC 1 — start first

python3 main_road.py --vms 2 --machine 2    # PC 2

## 7.2  Car states in the dashboard

| Symbol | State | Meaning |
| --- | --- | --- |
| → / ↗ | DRIVING / OVERTAKING | Normal forward movement |
| ▼ | BRAKING | Hard brake event |
| ↖ / ↘ | LANE_CHANGE | Moving between lanes |
| ⇐ | YIELDING | Yielding to emergency vehicle |
| ⇒ | SHOULDER | Pulled onto shoulder for emergency |
| ⛔ | BROKEN_DOWN | Out of fuel or mechanical failure |
| G / R / P | Traffic light | Green / Red / Pedestrian (intersection only) |
| → / ↘ / ↖ | Maneuver icons (intersection) | Straight / Turn right / Turn left |

## 7.3  Stopping the simulation

Press Ctrl+C in the terminal. The sim saves logs and exits cleanly. All model weights and training data are preserved.